# SKANN Document A
Underwater Acoustics Basics
(Final Corrected Version with Diagram Placeholders)

Oravont Systems LLP

# Introduction

Underwater acoustics establishes the physical foundation required for interpreting ocean‐borne sound. For SKANN, this foundation supports synthetic waveform creation (Stage −1), preprocessing (Stage 0), and all downstream machine‐learning tasks involving tonal lines, broadband noise, and transients.

This document presents a complete and corrected treatment of:

- acoustic pressure, RMS energy, and intensity,

- sound pressure level (SPL) and the “+6 dB detectability” rule,

- the underwater reference pressure of 1 μPa,

- transmission loss (TL), especially TL=20log10r,

- difference between noise in a 1 Hz band and pressure PSD,

- the p(t) versus p2(t) modelling debate and final resolution,

- definitions, units, and symbols (to be reused in Documents B–D),

- placeholders for figures with authoritative reference links.

The content is self‐contained and written in a clean, structured, engineering‐grade style. All mathematics uses Pandoc‐compatible LaTeX (no TikZ, no math inside list labels).

# Acoustic Pressure

## Instantaneous acoustic pressure p(t)

Underwater sound consists of small fluctuations in pressure around the static hydrostatic pressure. The instantaneous acoustic pressure is

p(t),

measured in pascals (Pa). This is the quantity directly sensed by a hydrophone.

After digitisation at sampling frequency fs, the discrete‐time sequence is

p[n]=p(n Δt),  Δt=1fs, n=0,…,N−1.

This discrete waveform is the starting point for SKANN’s analysis and for synthetic ambient noise generation.

## Mean-square pressure and RMS pressure

Because p(t) oscillates around zero, its magnitude is represented using the mean-square pressure

p2=1T0Tp(t)2 dt,

and its square root, the root mean-square (RMS) pressure,

prms=p2.

In discrete form,

prms=1Nn=0N−1p[n]2.

RMS pressure (and its square) is the basis of SPL, band levels, PSD integration, and detectability rules.

# Underwater Noise Components

Underwater noise relevant to vessel classification has three principal components: tonal, broadband, and transient. Their physical origin and spectral structure influence how SKANN identifies vessels.

## Tonal components

Tonal lines arise from periodic mechanical motion such as:

- shaft rotation,

- gearbox mesh,

- generator harmonics,

- propeller blade rate.

The blade‐rate frequency is

fBR=B nshaft60,

where B is blades per propeller and nshaft is in rpm.

These lines act as stable identifiers.

## Broadband components

Broadband noise arises from hydrodynamic turbulence, flow noise, cavitation, and structural vibration. In a PSD, it appears as a smooth spectrum without sharp peaks. It strongly depends on vessel speed.

## Transient components

Transient noise includes:

- cavitation onset,

- manoeuvring events,

- mechanical impacts,

- impulsive hydrodynamic events.

These appear as short‐duration bursts in time and spectrograms.

## Ambient noise

The ocean contains persistent background noise from:

- wind and waves,

- distant shipping,

- marine life,

- rainfall,

- geophysical activity.

Ambient noise spectra are handled in detail in Document B (Knudsen, Wenz, Kießling).

# Sound Pressure Level, Reference Pressure, and Detectability

## Reference pressure 1 μPa

Underwater acoustics uses

pref=1 μPa=10−6 Pa

as the reference pressure. This convention:

- reflects the high acoustic impedance of seawater,

- is used in all hydrophone calibrations,

- matches Knudsen and Wenz ocean-noise spectra (dB re 1 μPa or dB re 1 μPa​2/Hz).

## Sound Pressure Level (SPL)

SPL is defined as:

SPL=20log10prmspref [dB re 1 μPa].

Because prms2 is proportional to acoustic intensity, using 20log10 on pressure is equivalent to 10log10 on power.

## The +6 dB detectability rule (Corrected)

Detectability in underwater acoustics is expressed as a **difference of levels**, not by raw SNR of power ratios.

Let:

- Lsig = signal level (dB),

- Lnoise = noise level (dB).

Detectability threshold used in SKANN:

Lsig−Lnoise≈6 dB.

This corresponds to the signal’s RMS pressure amplitude being about a factor of 2 (power factor of 4) above ambient in the same band. A +6 dB gap gives clear visibility of tonals and many broadband features.

# Spectral Analysis: PSD, Bandpower, and 1 Hz Noise Bands

## Discrete Fourier Transform

Given N samples at sampling frequency fs, the DFT is:

P[k]=n=0N−1p[n]e−j2πkn/N.

Frequency bin:

fk=kfsN,  Δf=fsN.

## Power Spectral Density (PSD)

The (one-sided) pressure PSD Spp(f) represents mean-square pressure per unit bandwidth (Pa​2/Hz). A periodogram estimate (ignoring window effects) is:

Spp(fk)≈2N2|P[k]|2Δf (k>0).

Band-limited mean-square pressure over [f1,f2]:

p2[f1,f2]=f1f2Spp(f) df.

Band level:

L[f1,f2]=10log10p2[f1,f2]pref2 [dB re 1 μPa].

## Noise level in 1 Hz vs PSD (Corrected)

Many ambient-noise curves (Knudsen, Wenz) publish noise level:

NL(f) [dB re 1 μPa​2/Hz].

Convert back to PSD as:

Spp(f)=(1 μPa2/Hz) 10NL(f)/10.

Total noise in a 1 Hz band centred at f0:

p21Hz(f0)≈f0−0.5f0+0.5Spp(f) df.

Interpretation:

- Spp(f) is a density in Pa​2/Hz,

- integrating over 1 Hz yields total noise power in that band.

They are related but not identical. SKANN uses Spp(f) as the primary model input.

# Propagation and Transmission Loss

## Spherical spreading

For a point source radiating acoustic power Pacoustic, the intensity at distance r is:

I(r)=Pacoustic4πr2.

For a plane wave in a lossless medium:

I(r)=prms(r)2ρc.

Thus:

prms(r)∝1r.

Reference: USRD – Spherical Spreading Model: https://oalib-acoustics.org/intro/propagation/spherical

Spherical spreading geometry.

## Transmission Loss (Corrected)

Transmission loss relative to r0 is:

TL(r)=20log10rr0.

For r0=1 m:

TL(r)=20log10(r).

This is the correct spreading regime for early-stage SKANN synthetic waveforms.

## Source Level and Received Level

Source level:

SL=20log10prms(1 m)pref [dB re 1 μPa @ 1 m].

Received level at distance r:

RL(r)=SL−TL(r).

Example: At r=100 m:

TL≈40 dB,  RL≈SL−40 dB.

# RMS Pressure, p2, and Acoustic Intensity

## Mean-square pressure and RMS (Recap)

p2=1T0Tp(t)2 dt,  prms=p2.

Because p(t) oscillates around zero, only p2 carries energy information.

## Acoustic intensity

Intensity:

I=prms2ρc [W/m​2].

Thus intensity ∝prms2.

Reference: Acoustical Society of America — Acoustics Primer: https://acousticalsociety.org/acoustics-101/

Relationship between p(t), p2(t), RMS pressure, and intensity.

# The p(t) vs p2(t) Debate (Final SKANN Resolution)

## The tempting but incorrect approach

A naive idea:

> “Since levels depend on p2, maybe we can model p2(t) directly.”

This is incorrect because:

- Squaring introduces components at 0 Hz and 2f0 for each tone.

- Broadband noise becomes non-Gaussian and spectrally distorted.

- FFT[p2(t)] has no physical meaning.

- PSD reconstruction using p2(t) is impossible.

## Correct method: Synthesis from Spp(f) to p(t)

The correct SKANN procedure is:

- Choose fs, N, Δf.

- Obtain target PSD Spp(f).

- Compute amplitude spectrum:

|P(fk)|=Spp(fk) Δf.

- Assign random phases:

ϕk∼Uniform(0,2π).

- Form complex spectrum: P(fk)=|P(fk)|ejϕk.

- Impose Hermitian symmetry.

- IFFT → p[n].

- Scale to required RMS.

Reference: MIT OCW — Random Signals and Spectra: https://ocw.mit.edu/courses/6-011-signals-systems/

PSD →Spp(f) → random phase → IFFT → p(t) synthesis path.

## Final SKANN resolution

- Always synthesise **pressure** waveforms p(t).

- Use p2(t) only for energy and level calculations.

- PSD is the fundamental spectral quantity.

# Spectral Analysis Figures (Inserted in Context)

## Figure: Tonal vs Broadband Noise Spectrum

Reference: NOAA Ocean Acoustics: https://ocean.noaa.gov/science/ocean-sound/acoustics-101/

Illustration of tonal line above broadband noise floor.

## Figure: 1 Hz Noise Band vs PSD

Reference: National Instruments — PSD Introduction: https://www.ni.com/en/support/documentation/supplemental/06/power-spectral-density.html

Difference between PSD (Pa​2/Hz) and noise in a 1 Hz band.

# FFT Grid

FFT bin spacing and sampling:

fk=kfsN,  Δf=fsN.

Reference: MathWorks — FFT Fundamentals: https://in.mathworks.com/help/matlab/ref/fft.html

Frequency-domain structure: bins, resolution, and sampling grid.

# How SKANN Uses These Acoustic Foundations

- Tonals → selective convolutional filters.

- Broadband → CNN receptive fields.

- Transients → high-temporal-resolution kernels.

- PSD slope → governs SSL pretraining difficulty.

- RMS/SPL → set SNR and vessel–ambient levels.

- TL → range simulation for synthetic data.

- Spp(f) → the core ambient noise input.

- p(t) → the universal input to SKANN’s models.

Document B introduces ambient noise models (Knudsen/Wenz/Kießling). Document C formalises DSP and sampling. Document D synthesises wideband ambient noise.

# Appendix A — List of Symbols (For Documents A–D)

This appendix defines all symbols used consistently across Documents A, B, C, and D. All definitions are Pandoc-safe (no math inside item labels).

## Time-Domain and Sampling

- t: continuous time variable [s].

- n: sample index (integer).

- fs: sampling frequency [Hz].

- Δt: sampling interval 1/fs [s].

- N: number of samples per window.

- T: window duration T=N/fs [s].

## Acoustic Pressure

- p(t): instantaneous acoustic pressure [Pa].

- p[n]: discrete-time pressure sample [Pa].

- p2: mean-square acoustic pressure [Pa​2].

- prms: RMS acoustic pressure [Pa] or [μPa].

- pref: reference pressure (1 μPa).

## Levels and Noise Quantities

- SPL: sound pressure level, 20log10(prms/pref) [dB re 1 μPa].

- NL(f): noise level [dB re 1 μPa​2/Hz].

- Spp(f): pressure PSD [Pa​2/Hz].

- Band level: L[f1,f2] [dB re 1 μPa].

- SNR: signal-to-noise ratio (dB): Lsig−Lnoise.

## Propagation

- r: range (distance from source) [m].

- r0: reference range (typically 1 m) [m].

- TL: transmission loss [dB].

- SL: source level [dB re 1 μPa @ 1 m].

- RL: received level [dB re 1 μPa].

## Medium Properties

- ρ: density of seawater (≈ 1025 kg/m​3).

- c: sound speed (≈ 1500 m/s).

- I: acoustic intensity [W/m​2].

## Frequency-Domain Quantities

- f: frequency [Hz].

- fk: DFT bin frequency (kfs/N).

- Δf: frequency resolution (fs/N).

- P(f): Fourier transform of p(t).

- P[k]: DFT of p[n].

## Vessel and Tonal Features

- B: number of propeller blades.

- nshaft: shaft speed [rpm].

- fBR: blade-rate frequency [Hz].

## Synthetic Waveform Generation

- ϕk: random phase for bin k (uniform in [0,2π)).

- IFFT: inverse discrete Fourier transform.

# Notes on Pandoc Compatibility

To ensure that pandoc Document_A.tex -o Document_A.docx works cleanly:

- No TikZ or PGF figures.

- No math expressions inside list labels.

- All math environments ($,

- , equation) are properly closed.

- URLs inserted using \url{}.

- No custom LaTeX packages beyond amsmath, amssymb, geometry, setspace, and url.