# SKANN Document C
DSP and Sampling Logic for SKANN Stage 0
(Final Structured Draft)

Oravont Systems LLP

# Introduction

Document C defines the digital signal processing (DSP) conventions used in the SKANN project. These conventions ensure that all synthetic and recorded underwater signals are:

- comparable in sampling rate,

- comparable in amplitude scale,

- represented in a consistent spectral framework,

- compatible with machine-learning front-ends,

- correctly aligned with Documents A (acoustics) and B (ambient noise).

SKANN Stage 0 (Preprocessing & Standardisation) uses the definitions in this document to:

- resample incoming audio,

- normalise amplitude,

- compute FFT size and bin spacing,

- segment data,

- prepare tensors for the learned filterbank (Stage 1).

# Sampling Frequency

The sampling frequency fs determines the highest representable frequency and the time resolution. For SKANN, underwater signals are band-limited to:

0≤f≤32 kHz.

Thus a sampling rate of:

fs=32768 Hz

is chosen because:

- it satisfies Nyquist for 0–32 kHz,

- it is a power-of-two multiple (32k × 1.024),

- it enables efficient FFT computation,

- it is compatible with embedded / hydrophone ADCs.

# Nyquist Frequency

The highest representable frequency is:

fNyquist=fs2.

For fs=32768 Hz:

fNyquist=16384 Hz.

Since ambient-noise PSD is defined up to 32 kHz, SKANN uses frequency mirroring for synthetic wideband signals in Stage −1 (Document D), but Stage 0 only processes real hydrophone data band-limited to 0–16 kHz.

# FFT Size and Frequency Resolution

Given FFT size N and sampling rate fs, the frequency resolution is:

Δf=fsN.

Typical FFT sizes in SKANN:

N∈{512, 1024, 2048, 4096, 8192}.

Examples:

N=4096⇒Δf≈8 Hz,

N=8192⇒Δf≈4 Hz.

Higher N gives finer spectral detail but increases CPU/GPU load.

The FFT bin frequencies are:

fk=kfsN,  k=0,…,N−1.

# Windowing and Frame Segmentation

Before computing the FFT, each frame of length N samples is multiplied by a window function w[n] to reduce spectral leakage.

Common choices:

- Hann window,

- Hamming window,

- Blackman window.

The windowed frame is:

pw[n]=p[n] w[n],  n=0,…,N−1.

## Overlap and Hop Size

For continuous signals, frames overlap to ensure good time resolution:

hop=N−overlap.

Typical SKANN values:

N=4096, overlap=50%⇒hop=2048.

This maintains smooth spectrograms and avoids information loss.

# DFT Conventions and One-Sided Spectrum

Given the windowed frame pw[n], the DFT is:

P[k]=n=0N−1pw[n] e−j2πkn/N.

For real-valued input:

P[k]=P*[N−k],  k>0.

Therefore, only indices 0≤k≤N/2 contain unique information.

# Power Spectral Density (PSD) Estimation

PSD expresses mean-square pressure per unit bandwidth (Pa​2/Hz).

## Periodogram

The periodogram estimate is:

Spp(fk)=2N U|P[k]|2fs,  k=1,…,N/2−1.

Where U is the window power:

U=1Nn=0N−1w[n]2.

The factor of 2 accounts for folding negative frequencies into positive frequencies (one-sided PSD).

## Welch PSD

To obtain a smoother PSD, SKANN optionally uses Welch’s method:

- Split data into overlapping frames.

- Apply window to each frame.

- Compute periodogram for each frame.

- Average periodograms:

SppWelch(fk)=1Mm=1MSpp(m)(fk).

This reduces variance of the PSD estimate but slightly increases bandwidth.

# Bandpower Computation

Given a PSD Spp(f), the mean-square pressure in band [f1,f2] is:

p2[f1,f2]=f1f2Spp(f) df.

Discrete approximation:

p2[f1,f2]≈fk∈[f1,f2]​Spp(fk) Δf.

Band level in dB:

L[f1,f2]=10log10p2[f1,f2]pref2.

# Hermitian Symmetry and Signal Reconstruction

For reconstruction or synthesis (used in Document D), SKANN uses Hermitian symmetry to enforce real-valued time-domain signals:

P[k]=P*[N−k],  1≤k≤N/2−1.

With:

P[0]∈R,  P[N/2]∈R.

This guarantees that the inverse FFT:

p[n]=IFFT{P[k]}

produces real-valued samples p[n]∈R.

# Amplitude Processing and Normalisation

Before sending any waveform into SKANN Stage 0, the amplitude must be standardised so that all datasets (synthetic and real) are comparable.

## DC Offset Removal

The first step is to ensure that:

1Nn=0N−1p[n]=0.

DC offset is removed by:

p[n]←p[n]−μ,  μ=1Nn​p[n].

This prevents artificial low-frequency artefacts.

## Peak Normalisation

Peak-normalised signal:

ppeaknorm[n]=p[n]max|p[n]|.

Peak normalisation makes all recordings fit within [−1,1] and is used for preliminary sanity checks.

## RMS Normalisation

More physically meaningful is RMS-based scaling:

prmsnorm[n]=p[n]prms.

This sets the continuous-time RMS to:

prmsnorm=1.

Synthetic training data optionally uses target RMS values corresponding to:

SPLtarget=20log10prms,targetpref.

# Resampling Logic

All signals must be resampled to the SKANN standard sampling rate:

fs=32768 Hz.

## Anti-Alias Filtering

Before down-sampling by a factor R:

fsold→fsnew,  R=fsoldfsnew,

an anti-alias filter with passband:

0≤f≤fsnew2

must be applied.

## Upsampling

Upsampling inserts zeros between samples and applies a reconstruction filter:

p↑R[n]=p[n/R],n=0,R,2R,…0,otherwise

followed by low-pass filtering.

## Polyphase Resamplers

In practice, SKANN uses library polyphase filters (SRC / resampy) for high-quality resampling with minimal distortion.

# Silence Trimming and Energy Gating

SKANN avoids frames with very low energy, since they harm SSL pretraining and classification.

A short-time energy gate is:

Em=n=0N−1pm[n]2.

Frames satisfying:

Em<τ

(where τ is a percentile threshold) are removed.

# Segmentation and Padding

Segments are extracted with fixed duration:

Tseg=Nfs.

Typical values:

N=4096, 8192, 16384.

Padding rules:

- pad with zeros if the last segment is short,

- pad symmetrically for CNNs that require even-length inputs.

# Tensor Preparation for SKANN

After segmentation, each frame of length N becomes a tensor:

x∈R1×N.

A batch is:

X∈RB×1×N,

where B is batch size.

For STFT-based learners (optional):

Xspec∈RB×F×T,

where:

F=N/2+1  (frequency bins),

T=number of frames.

# Stage 0 Preprocessing Pipeline (Summary)

Stage 0 prepares raw or synthetic signals into a uniform digital format for ingestion by the learned filterbank (Stage 1). The steps are:

- DC removal: subtract the mean to stabilise the low-frequency spectrum.

- Resampling: convert all signals to fs=32768 Hz with anti-alias filtering.

- Amplitude normalisation: peak-normalise for safety, RMS-normalise for consistency.

- Segmentation: extract segments of length N with 50% overlap.

- Windowing: apply Hann/Hamming window before FFT.

- Silence gating: remove frames whose short-time energy Em is below threshold.

- Tensor formation: final waveform tensor:

X∈RB×1×N.

- (Optional) STFT: for spectrogram-based models:

Xspec∈RB×F×T.

# Stage 0 Flowchart (Placeholder)

Reference: SKANN internal DSP framework.

SKANN Stage 0 preprocessing pipeline (conceptual).

# Appendix A — Unified List of Symbols

The following symbols match Documents A and B to maintain consistency.

## Sampling and Frames

- fs: sampling frequency [Hz].

- Δt: sampling interval 1/fs [s].

- N: samples per frame.

- T=N/fs: frame duration [s].

- hop: frame advance in samples.

## Frequency-Domain Quantities

- fk: FFT bin frequency.

- Δf: FFT bin spacing [fs/N].

- P[k]: DFT of windowed frame.

- Spp(f): pressure PSD [Pa​2/Hz].

- SppWelch: Welch PSD estimate.

## Amplitude and Energy

- p[n]: discrete-time pressure.

- prms: RMS amplitude.

- pref: reference amplitude (1 μPa).

- Em: frame energy.

## Synthesis/Reconstruction

- ϕk: random phase.

- IFFT: inverse discrete Fourier transform.

- |P(fk)|: amplitude spectrum.

# Pandoc Compatibility Notes