# SKANN Document D
Ambient Noise Synthesis (Stage −1)
Generating Time-Domain Waveforms from Spectral Models

Oravont Systems LLP

# Introduction

Document D describes the full methodology for generating time-domain ambient-noise waveforms p(t) from the blended and digitised PSD Spp(f) produced in Document B. This spectral-to-time conversion forms the basis of SKANN Stage −1, where synthetic ocean-noise waveforms are produced for:

- pretraining the SSL backbone,

- data augmentation,

- controlled SNR mixing with vessel signatures,

- benchmarking detection sensitivity.

Document D covers all required steps:

- FFT grid construction,

- amplitude spectrum computation,

- random phase generation,

- Hermitian symmetry enforcement,

- inverse FFT,

- amplitude normalisation,

- multi-segment and multi-channel synthesis.

# Overall Synthesis Pipeline

The synthesis process maps a PSD to a time-domain signal using the following sequence:

- Digitised PSD (from Document B):

Spp(fi) (0≤fi≤32 kHz).

- Assign FFT frequency grid corresponding to sampling rate fs and FFT size N:

fk=kfsN.

- Interpolate Spp(f) onto {fk}.

- Compute amplitude spectrum:

|P(fk)|=Spp(fk) Δf.

- Assign random phase:

ϕk∼Uniform(0,2π).

- Construct complex spectrum:

P(fk)=|P(fk)| ejϕk.

- Impose Hermitian symmetry to ensure p(t) is real.

- Inverse FFT:

p[n]=IFFT{P(fk)}.

- RMS scaling to match physical sea-state levels.

# Diagram Placeholder: Complete Synthesis Flow

Reference: SKANN synthetic noise generator.

Conceptual pipeline from PSD to time-domain waveform.

# Construction of the FFT Frequency Grid

Let the chosen sampling rate and FFT size be:

fs=32768 Hz,  N∈{4096, 8192, 16384}.

The frequency resolution is:

Δf=fsN.

Thus the k-th discrete frequency is:

fk=kfsN,  k=0,…,N−1.

For real-valued signals, only

0≤k≤N2

contain unique frequencies.

# Interpolation of the PSD onto the FFT Grid

Document B produces a log-spaced set of PSD points:

Spp(fi),  i=1,…,Nf,

where fi may extend to 32 kHz.

To synthesise time-domain noise, the PSD must be evaluated on the FFT grid {fk}:

Spp(fk)=interpSpp(fi),fi→fk.

Interpolation is performed in log-frequency:

log10fi→log10Spp(fi),

because ambient noise follows power-law relationships.

# Amplitude Spectrum from PSD

A PSD expresses power per unit bandwidth:

Spp(fk)=|P(fk)|2Δf.

Thus:

|P(fk)|=Spp(fk) Δf.

This ensures that:

k​|P(fk)|2≈k​Spp(fk)Δf

which matches the correct total mean-square pressure.

## Worked Example: Sea State 3 at 1 kHz (Δf = 1 Hz)

Consider Sea State 3 at f0=1 kHz. From Document B, the noise level is:

NL(1 kHz)=64

Units: dB re 1 μPa2/Hz.

Convert this to linear PSD:

Spp(1 kHz)=1064/10 μPa2/Hz

Numerically:

2.51×106 μPa2/Hz=2.51×10−6 Pa2/Hz

With Δf=1 Hz:

|P(f0)|=Spp(f0) Δf=2.51×10−6=1.58×10−3

Thus the one-sided FFT magnitude at 1 kHz is:

|P(1 kHz)|=1.6×10−3

## Time-Domain Interpretation of a Single Component

A positive-frequency bin and its conjugate form a real cosine with **twice** the one-sided amplitude:

ptone(t)=2|P(1 kHz)|cos(2π⋅1000 t+ϕ0)

Numerically:

ptone(t)=3.2×10−3cos(2π⋅1000 t+ϕ0)

# Random Phase Assignment

Ambient noise has no deterministic phase, so each bin is assigned

ϕk∼Uniform(0,2π)

and

P(fk)=|P(fk)| [cos(ϕk)+jsin(ϕk)]

## Worked Example: Random Phase at 1 kHz

Amplitude from previous section:

|P(1 kHz)|=1.6×10−3

Step 1: draw u:

u=0.37

Step 2: convert to phase:

ϕ0=2πu=2.325 rad

Step 3: construct the complex spectrum value:

cos(2.325)=−0.688,  sin(2.325)=0.726

Thus:

P[1000]=1.6×10−3 (−0.688+j 0.726)

## Hermitian Symmetry: Explicit Conjugate Assignment

For a real time-domain waveform, the FFT must satisfy:

P[N−k]=P[k]

With N=32768 and k0=1000:

N−k0=31768

Compute the conjugate:

P[1000]=1.6×10−3 (−0.688−j 0.726)

Assign to symmetric bin:

P[31768]=P[1000]

## Resulting Time-Domain Tone

The pair {1000,31768} produces:

ptone(t)=3.2×10−3cos(2π⋅1000 t+ϕ0)

At t=0:

ptone(0)=3.2×10−3cos(2.325)=−2.20×10−3 Pa

At t=1 ms:

ptone(0.001)=3.2×10−3cos(2π+2.325)=−2.20×10−3 Pa

# Hermitian Symmetry Enforcement

To guarantee real-valued samples:

P[k]=P*[N−k],  1≤k≤N/2−1

and special bins:

P[0]∈R,  P[N/2]∈R

After symmetry is imposed, the spectrum is ready for the inverse FFT.

# Inverse FFT

The synthetic time-domain waveform is obtained via:

p[n]=IFFT{P(fk)},  n=0,…,N−1.

Because Hermitian symmetry is satisfied, p[n] is guaranteed to be real:

p[n]∈R.

The waveform retains the exact PSD shape defined in Document B and is ready for RMS-level scaling.

# RMS Scaling and SPL Matching

The raw IFFT output has correct spectral shape but arbitrary amplitude. To match a desired sea-state or absolute level, the waveform is scaled to a target RMS value.

Let the generated signal have RMS:

prms,gen=1Nn=0N−1p[n]2.

Let the desired RMS correspond to a physical SPL:

SPLtarget=20log10​prms,targetpref,

with reference pressure pref=1 μPa.

Scale the signal:

p[n]←p[n] prms,targetprms,gen.

This is applied once only, after the IFFT.

# Generating Multi-Second Noise Segments

The previous steps generate a single noise frame of length N samples. To produce long-duration noise (e.g., 2–30 seconds), multiple IFFTs are concatenated.

A frame produces duration:

Tframe=Nfs.

Example:

N=32768, fs=32768 Hz ⇒ Tframe=1 s.

A multi-second duration T requires:

M=TTframe

frames.

# Avoiding Discontinuities Between Frames

Simply concatenating IFFT outputs can produce discontinuities at frame boundaries because each frame uses independent random phase. SKANN uses one of two solutions:

## Method 1: Overlap–Add (Recommended)

Each frame is windowed with a smooth taper (50% overlap):

pw(m)[n]=p(m)[n] w[n].

Frames are placed into the output with:

hop=N2.

The final signal is:

plong[n]=m=0M−1pw(m)[n−m hop].

Window choice ensures:

w[n]2+w[n+N/2]2=1,

so the energy is preserved.

## Method 2: Correlated Phase Propagation

To ensure smooth phase continuation, the random phases can be generated as:

ϕk(m)=ϕk(m−1)+δϕk,

where δϕk is small and random. This preserves continuity at the cost of reduced randomness.

SKANN uses Method 1 because it is simpler, robust, and produces natural noise.

# Long-Duration Synthesis (Minutes or Hours)

For extended-duration acoustic backgrounds (e.g., simulation of hydrophone streams):

- use block-based OLA synthesis,

- randomise seeds per block,

- enforce continuity using Method 1,

- periodically re-normalise RMS every 10–20 minutes.

Output lengths of 1 hour or more are straightforward.

# Multi-Channel Synthesis (Optional)

For hydrophone arrays, including vector sensors, SKANN supports multi-channel noise synthesis.

## Independent-Channel Noise

If channels are assumed uncorrelated:

pc[n]=IFFT|P(fk)|ejϕk,c,

with independent phases ϕk,c.

## Spatially Correlated Noise

If physical spatial correlations are required (e.g., HAVS dipole sensors):

Pc(fk)=Hc(fk) Pref(fk),

where:

- Pref(fk) is the base noise spectrum,

- Hc(fk) is the channel’s spatial response.

Examples:

Hvertical(fk)=jfkf0,  Hhorizontal(fk)=cosθ fkf0.

These models allow SKANN to simulate:

- tri-axial vector sensors,

- hydrophone arrays,

- correlated ambient-noise fields.

# Diagram Placeholder: Multi-Segment OLA

Reference: SKANN long-duration synthesis.

Overlap–Add synthesis of long ambient-noise waveforms.

# Complete Synthesis Algorithm (Pseudocode)

This section summarises the full procedure in algorithmic form. The notation matches Documents B and C.

## Input

- Digitised PSD Spp(fi) over 0–32 kHz (Document B)

- Sampling rate fs=32768 Hz

- FFT size N (typically 32768 for Δf=1 Hz)

- Desired duration T

- Target RMS level prms,target

## Output

Time-domain ambient-noise waveform plong[n].

## Pseudocode

1. Construct FFT frequencies:
       f_k = k * f_s / N,       k = 0 .. N/2

2. Interpolate PSD onto FFT grid:
       Spp_fk = interp( Spp_fi → f_k )

3. Compute amplitude spectrum:
       A_k = sqrt( Spp_fk * Δf )

4. Generate random phases:
       φ_k ~ Uniform(0, 2π)

5. Construct complex spectrum:
       P[k] = A_k * exp(j φ_k)

6. Impose Hermitian symmetry:
       P[N-k] = conj(P[k])

7. Inverse FFT:
       p_frame = IFFT(P)

8. RMS-scale the frame:
       p_frame ← p_frame * (p_rms_target / p_rms_generated)

9. For long durations:
       - Generate M frames
       - Apply Hann window
       - Overlap-add with hop = N/2

10. Output:
       p_long[n]

# Integration with Stage 0 (Document C)

When plong[n] is produced:

- Resampling is not needed because synthesis already uses fs=32768 Hz.

- DC offset is already zero (expected from IFFT), but Stage 0 verifies:

p[n]←p[n]−1Nn​p[n].

- Amplitude normalisation in Stage 0 is typically disabled for synthetic noise, since Stage −1 has applied the correct RMS.

- Segmentation into fixed-length frames (e.g. 4096 samples) is performed exactly as for real hydrophone data.

- Silence trimming is skipped; noise has no silent frames.

Thus the output of Document D becomes a valid input for Stage 0 and then for the learned filterbank in Stage 1.

# Diagram Placeholder: Full Stage −1 to Stage 0 Interface

Integration of synthetic noise into the SKANN processing chain.

# Appendix A — Unified List of Symbols (Documents A–D)

## Spectral Quantities

- fk: FFT bin frequency [Hz]

- Δf: frequency resolution [fs/N]

- Spp(f): pressure PSD [Pa​2/Hz]

- |P(fk)|: amplitude spectrum

- ϕk: random phase

## Time-Domain Quantities

- p[n]: discrete-time waveform

- prms: RMS pressure

- pref=1 μPa: reference pressure

## Noise Model Components

- NL(f): noise level [dB re 1 μPa​2/Hz]

- SS: sea state (0–6)

- CLF,CMF,CHF: PSD scaling

# Pandoc Compatibility Notes

- No TikZ or pgfplots used.

- Figures implemented using figure with boxed placeholders.

- Math expressions closed properly for DOCX conversion.

- Standard packages only: amsmath, amssymb, geometry, setspace, url.