# Parametric Construction of Sea–Noise Spectra from Digitised Knudsen Curves

# 1. Introduction

This document describes the procedure followed to construct a continuous, piecewise parametric model of ocean ambient noise suitable for SKANN simulations. The construction begins with digitisation of reference Knudsen curves, continues with segmentation of the spectral domain into physically meaningful sub–bands, and concludes with log–frequency regression and turbulence modelling. The resulting representation is fully analytic, continuous at all band junctions, and suitable for generating synthetic waveforms or coloured noise.

Placeholder for Knudsen Curve Figure:

# 2. Digitisation of Reference Curves

The Knudsen chart provides spectral noise levels in dB re 1 μPa​2/Hz for different sea states. Since this information is typically available as a plot, WebPlotDigitizer (WPD) was used to extract numerical points for sea states SS0, SS1, SS3, and SS6 in the frequency range 10 Hz to 100 kHz.

## 2.1 WebPlotDigitizer Procedure

- Axes were configured as: X–axis logarithmic, Y–axis linear.

- Two calibration points were selected on the X–axis (for example 10 Hz and 100 kHz).

- Two calibration points were selected on the Y–axis (for example 20 dB and 100 dB).

- Each sea–state curve was isolated using colour masking.

- Automatic curve extraction was used to obtain a set of (index, frequency, noise level) values.

- The resulting CSV files were exported without headers.

Each file contained three columns of the form

(i, f_Hz, NL_dB).

# 3. Loading and Plotting the Digitised Curves

The CSV files are loaded and plotted using Python. The example below shows how the four sea states SS0, SS1, SS3, and SS6 are read and displayed.

import pandas as pd
import matplotlib.pyplot as plt

# Load digitised Knudsen curves for four sea states.
# Each file is: index, frequency_Hz, noise_level_dB (no header row).
SS0 = pd.read_csv("SS0CSV.txt", header=None, names=["idx","f","NL"])
SS1 = pd.read_csv("SS1CSV.txt", header=None, names=["idx","f","NL"])
SS3 = pd.read_csv("SS3CSV.txt", header=None, names=["idx","f","NL"])
SS6 = pd.read_csv("SS6CSV.txt", header=None, names=["idx","f","NL"])

# Ensure increasing frequency order for safety and interpolation.
for df in [SS0, SS1, SS3, SS6]:
    df.sort_values("f", inplace=True)

# Plot all four digitised curves on a semilog frequency axis.
plt.figure(figsize=(12,8))
plt.semilogx(SS0["f"], SS0["NL"], label="SS0")
plt.semilogx(SS1["f"], SS1["NL"], label="SS1")
plt.semilogx(SS3["f"], SS3["NL"], label="SS3")
plt.semilogx(SS6["f"], SS6["NL"], label="SS6")
plt.xlabel("Frequency (Hz)")
plt.ylabel("Noise Level (dB re 1 µPa²/Hz)")
plt.title("Digitised Knudsen Curves for Four Sea States")
plt.grid(True, which='both', linestyle='--', alpha=0.4)
plt.legend()
plt.show()

Placeholder for Digitised Curves Plot:

Superposition of the four curves confirms the correctness and smoothness of the digitised data across the entire 10 Hz to 100 kHz range.

# 4. Motivation for Spectral Segmentation

Inspection of the digitised Knudsen sea–noise spectra reveals that no single functional expression accurately models the entire 10 Hz–100 kHz range. Instead, distinct physical mechanisms dominate separate frequency regions, and the spectrum therefore exhibits piecewise power–law behaviour.

The principal regions observed in the Knudsen, Wenz, and Kießling curves are:

- Hydrodynamic turbulence below approximately 40–60 Hz, characterised by a steep downward slope in NL versus log10f.

- Low–frequency (LF) shipping and wind noise, beginning at the turbulence–LF intersection fTI and extending to about 200 Hz, with a comparatively shallow spectral slope.

- Mid–frequency (MF) shoulder, typically spanning 200–800 Hz in classical presentations. For SKANN, the MF plateau used for parametrisation is restricted to 200–500 Hz, where the digitised SS3 data show the flattest spectral region.

- High–frequency (HF) wind–sea band, beginning near 500 Hz and extending to 100 kHz, following an approximately straight line with slope −20 to −26 dB per decade.

These regions correspond to standard physical interpretations of the Knudsen curve and also yield models that are linear in (log10f, NL) space.

Earlier drafts of this document used a 300–800 Hz MF segmentation; however, enforcing an HF join at 300 Hz is physically incorrect and led to an artificial kink. The HF line that best fits the digitised SS3 data intersects the MF region near 500 Hz, not 300 Hz. Therefore the final, physically motivated boundaries adopted for SS3 are

fTI,  200 Hz,  500 Hz,

with a true MF plateau between 200–500 Hz and a single straight HF line starting at 500 Hz and continuing to 100 kHz.

This segmentation provides a smooth, stable basis for deriving the parametric SS3 model used in subsequent sections.

# 5. Log–Frequency Regression and Coefficient Derivation

The Knudsen sea–noise spectrum exhibits approximate power–law behaviour within distinct frequency regions. A power law of the form

S(f)=Cfα

becomes linear under the noise–level mapping

NL(f)=10log10S(f)=10 α log10(f)+10log10(C).

Thus each spectral region may be represented by the affine model

NL(f)=a log10(f)+b,

where (a,b) are obtained either by numerical regression (HF and LF regions) or by enforcing smooth continuity at region boundaries (MF and turbulence regions).

For the corrected SS3 parametrisation, the four regions are:

Turbulence:10 Hz≤f<fTI,Low frequency (LF):fTI≤f<200 Hz,Mid–frequency (MF) plateau:200 Hz≤f≤500 Hz,High frequency (HF):500 Hz≤f≤100 kHz.

The goal of this section is to derive the parameters (a,b) for the LF and HF regions directly from the digitised WebPlotDigitizer CSV files, and to use continuity conditions to determine the MF plateau level and the turbulence intersection frequency fTI.

## 5.1 Least–Squares Determination of HF and LF Coefficients

To determine (a,b) for the HF and LF regions, we fit the affine model

NL(f)=a log10(f)+b

to the digitised samples (fi,NLi).

Let

xi=log10(fi),  yi=NLi.

Then the regression model becomes

yi=a xi+b+εi.

This leads to the linear system

Aθ=y,  A=x11x21⋮⋮xN1,  θ=ab.

The least–squares solution is

θ=(ATA)−1ATy,

computed numerically using the QR–based routine numpy.linalg.lstsq. This formulation solves Aθ=y; it does not use [a b]A.

#### Software Dependencies.

Only the numpy library is required for regression; pandas is used solely for reading and sorting CSV files. No SciPy or scikit–learn components are required.

#### Construction of the Design Matrix via np.vstack.

The design matrix A is assembled in Python using

A_hf = np.vstack([x_hf, np.ones_like(x_hf)]).T

which performs:

- construction of row vectors [x1,…,xN] and [1,…,1],

- vertical stacking into a 2×N matrix,

- transposition to obtain the N×2 design matrix A.

#### Least–Squares Solution via np.linalg.lstsq.

The HF regression is carried out as

a_HF, b_HF_raw = np.linalg.lstsq(A_hf, hf["NL"], rcond=None)[0]

which returns the unconstrained HF slope and intercept. A similar call on the LF subset yields (aLF,bLF).

#### Python Implementation Used.

Consistent with the corrected 200–500 Hz plateau and the single HF line starting at 500 Hz, the revised regression bands are:

# HF regression (3 kHz – 20 kHz)
hf = df[(df["f"] >= 3000.0) & (df["f"] <= 20000.0)]
x_hf = np.log10(hf["f"])
A_hf = np.vstack([x_hf, np.ones_like(x_hf)]).T
a_HF, b_HF_raw = np.linalg.lstsq(A_hf, hf["NL"], rcond=None)[0]

# LF regression (100 – 200 Hz)
lf = df[(df["f"] >= 100.0) & (df["f"] <= 200.0)]
x_lf = np.log10(lf["f"])
A_lf = np.vstack([x_lf, np.ones_like(x_lf)]).T
a_LF, b_LF = np.linalg.lstsq(A_lf, lf["NL"], rcond=None)[0]

The HF intercept bHF is further adjusted in Section 5.2 so that the HF line passes exactly through the MF plateau at 500 Hz.

## 5.2 Derivation of MF and Turbulence Coefficients

The MF band uses algebraic continuity. Let

x200=log10(200),  x500=log10(500).

Let NL(200) be interpolated from the SS3 digitised data in the neighbourhood of 200 Hz, and let NLHF(500) be obtained from the HF regression (Section 5.1). The MF line is then defined by

NLMF(f)=aMFlog10(f)+bMF,

where the coefficients follow from the two–point construction

aMF=NLHF(500)−NL(200)x500−x200,  bMF=NL(200)−aMF x200.

#### Turbulence line.

The turbulence segment is anchored using two physically motivated points on the Knudsen turbulence branch, for example

(10 Hz, 108 dB)  and  (40 Hz, 55 dB).

In NL versus log10(f) coordinates, the turbulence line takes the form

NLturb(f)=aturblog10(f)+bturb,

with coefficients

aturb=55−108log10(40)−log10(10),  bturb=108−aturb log10(10).

#### Intersection frequency.

The turbulence line meets the low–frequency (LF) line at the frequency ft defined by

aturblog10(ft)+bturb=aLFlog10(ft)+bLF,

so that

log10(ft)=bturb−bLFaLF−aturb,  ft=10 log10(ft).

The numerical values of aLF, bLF and the resulting ft are presented in Section 8, while Sections 9 and 10 describe the mid–frequency plateau and high–frequency segments constructed using these coefficients.

## 5.3 Summary of Numerical Coefficients (SS3)

The parametric SS3 model consists of four affine components in NL versus log10(f) space:

NL(f)=a log10(f)+b,

with coefficients derived either from least–squares regression (HF and LF), algebraic continuity (MF), or physically selected anchor points (turbulence). The final expressions are:

#### Turbulence region (10 Hz≤f<ft).

NLturb(f)=aturblog10(f)+bturb,

aturb=55−108log10(40)−log10(10),  bturb=108−aturb log10(10).

#### Low–frequency region (ft≤f<200 Hz).

NLLF(f)=aLFlog10(f)+bLF,

where the coefficients (aLF,bLF) are obtained by least–squares regression of the digitised SS3 points in the 100–200 Hz interval. Their numerical values appear in Section 8 together with the intersection frequency ft defined in Section 5.2.

#### Mid–frequency region (200 Hz≤f≤500 Hz).

NLMF(f)=aMFlog10(f)+bMF,

with

aMF=NLHF(500)−NL(200)log10(500)−log10(200),  bMF=NL(200)−aMF log10(200),

where NL(200) is interpolated from the digitised SS3 data and NLHF(500) is evaluated using the HF regression line. The resulting MF coefficients are reported in Section 9.

#### High–frequency region (500 Hz≤f≤100 kHz).

NLHF(f)=aHFlog10(f)+bHF,

where aHF is obtained via least–squares regression of the SS3 points from 500 Hz to 100 kHz. The intercept bHF is subsequently adjusted to ensure continuity with the MF segment at 500 Hz. The numerical HF values are presented in Section 10.

The next four sections (7–10) provide the explicit numerical coefficients for the turbulence, LF, MF, and HF regions, together with the corresponding frequency bounds defining the complete SS3 piecewise model.

# 6. Turbulence Region (10 Hz–ft)

The turbulence band models the steep decay in ambient sea noise at the lowest frequencies. Following the classical Knudsen formulation, this region is defined using two physically motivated anchor points,

(10 Hz, 108 dB),  (40 Hz, 55 dB),

which lie on the turbulence-dominated portion of the spectrum.

In NL versus log10(f) coordinates, the turbulence line is

NLturb(f)=aturblog10(f)+bturb.

#### Numeric coefficients.

Using the anchor points,

aturb=55−108log10(40)−log10(10)≈−33.10,  bturb=108.

The turbulence branch is taken to extend from 10 Hz up to the frequency ft at which it meets the low–frequency (LF) regression line. This intersection satisfies

NLturb(ft)=NLLF(ft),

and its numerical value is obtained in Section 7 once the LF coefficients are known.

# 7. Low–Frequency Region (ft–200 Hz)

The low–frequency (LF) region represents the transition from the steep turbulence slope into the mid–frequency shoulder. It is modelled as an affine function in log10(f),

NLLF(f)=aLFlog10(f)+bLF,  ft≤f<200 Hz.

#### Numeric coefficients from SS3 regression.

A least–squares regression of the digitised SS3 points between 100 Hz and 200 Hz yields

aLF≈13.30,  bLF≈36.29.

#### Intersection with the turbulence branch.

The lower boundary ft is obtained by solving

aturblog10(ft)+bturb=aLFlog10(ft)+bLF,

hence

log10(ft)=bturb−bLFaLF−aturb,  ft=10 log10(ft).

Substituting aturb≈−33.10, bturb=108 and aLF≈13.30, bLF≈36.29 gives

ft≈35.1 Hz.

Thus the LF region spans approximately 35 Hz to 200 Hz in the SS3 model.

# 8. High–Frequency Region (500 Hz–10 kHz)

The high–frequency (HF) region captures the wind–driven and breaking–wave noise components of the Knudsen spectrum. Over the range

500 Hz≤f≤10 kHz,

the digitised SS3 curve is well approximated by a straight line in NL versus log10(f) space.

Accordingly, the HF band is fitted using a least–squares regression over all SS3 points in this interval:

NLHF(f)=aHFlog10(f)+bHF.

#### Numeric coefficients (SS3 regression).

Regression over 500–10 kHz gives

aHF≈−16.22,  bHF≈110.65.

Evaluating the HF fit at 500 Hz gives

NLHF(500)=aHFlog10(500)+bHF≈66.87 dB.

This value forms the right–hand anchor for the mid–frequency (MF) segment constructed in Section 9.

# 9. Mid–Frequency Region (200–500 Hz)

The digitised SS3 spectrum exhibits a pronounced flattening between 200 Hz and 500 Hz, forming the characteristic mid–frequency (MF) shoulder. The MF segment is represented by an affine function in log10(f),

NLMF(f)=aMFlog10(f)+bMF,

constructed so as to meet the LF and HF lines exactly at 200 Hz and 500 Hz, respectively.

#### Boundary conditions.

Let

NLLF(200)=aLFlog10(200)+bLF≈66.89 dB,

and

NLHF(500)=aHFlog10(500)+bHF≈66.87 dB.

Thus the MF line interpolates between

(200 Hz, 66.89 dB) and (500 Hz, 66.87 dB).

#### Numeric coefficients.

With

x200=log10(200),  x500=log10(500),

the MF slope and intercept are

aMF=NLHF(500)−NLLF(200)x500−x200≈−0.033,

bMF=NLLF(200)−aMFx200≈66.96.

This produces a nearly flat MF shoulder, consistent with the SS3 curve, and guarantees continuity at both 200 Hz and 500 Hz.

# 10. Final Piecewise SS3 Model

Combining the turbulence, low–frequency, mid–frequency, and high–frequency components derived in Sections 6–9 yields the complete SS3 parametric noise model. In NL versus log10(f) coordinates, the spectrum is represented by the four affine segments:

NL(f)=aturblog10(f)+bturb,10 Hz≤f<ft,aLFlog10(f)+bLF,ft≤f<200 Hz,aMFlog10(f)+bMF,200 Hz≤f≤500 Hz,aHFlog10(f)+bHF,500 Hz≤f≤100 kHz.

The parameters (aturb,bturb), (aLF,bLF), (aMF,bMF), and (aHF,bHF) are summarised in Section 5.3, with their numerical values given in Sections 7–10.

This four–segment representation provides a continuous, physically interpretable approximation to the digitised SS3 Knudsen curve, suitable for synthetic waveform generation and downstream processing in the SKANN pipeline.

# 11. Verification by Superposition

To verify the accuracy of the parametric SS3 model, the four affine segments defined in Section 11 are superimposed onto the digitised SS3 data. The comparison plot serves two purposes:

- it demonstrates that each spectral region (turbulence, LF, MF, HF) provides a faithful local approximation to the corresponding portion of the SS3 curve, and

- it confirms that the imposed continuity conditions at ft, 200 Hz, and 500 Hz produce a smooth, visually seamless reconstruction of the overall spectrum.

A Python script for producing the superposition plot is provided in Section 13. The plotted result should show the SS3 points (scatter) and the four parametric branches (solid lines) with excellent alignment across all frequency ranges.

# 12. Python Code for Complete Parametric Fitting

The following Python script reproduces the SS3 parametric coefficients, constructs the four affine spectral branches, and generates a verification plot comparing the model with the digitised SS3 data.

import numpy as np
import pandas as pd
import matplotlib.pyplot as plt

# Load digitised SS3 CSV: columns = Index, Freq, NL
df = pd.read_csv("SS3CSV.txt", header=None, names=["Index","Freq","NL"])

# --- Turbulence coefficients (from anchor points)
a_turb = (55 - 108) / (np.log10(40) - np.log10(10))
b_turb = 108 - a_turb * np.log10(10)

# --- LF regression (100--200 Hz)
lf = df[(df["Freq"]>=100) & (df["Freq"]<=200)]
x_lf = np.log10(lf["Freq"].values)
A_lf = np.vstack([x_lf, np.ones_like(x_lf)]).T
a_LF, b_LF = np.linalg.lstsq(A_lf, lf["NL"].values, rcond=None)[0]

# --- Intersection f_t
log_f_t = (b_turb - b_LF) / (a_LF - a_turb)
f_t = 10**log_f_t

# --- MF coefficients
x200 = np.log10(200)
x500 = np.log10(500)
NL_200 = a_LF * x200 + b_LF

# Raw HF regression (500--100000 Hz)
hf = df[(df["Freq"]>=500) & (df["Freq"]<=100000)]
x_hf = np.log10(hf["Freq"].values)
A_hf = np.vstack([x_hf, np.ones_like(x_hf)]).T
a_HF_raw, b_HF_raw = np.linalg.lstsq(A_hf, hf["NL"].values, rcond=None)[0]

# MF right-end value from HF regression
NL_HF_500 = a_HF_raw * x500 + b_HF_raw

# MF slope and intercept
a_MF = (NL_HF_500 - NL_200) / (x500 - x200)
b_MF = NL_200 - a_MF * x200

# Final HF intercept ensuring continuity at 500 Hz
b_HF = NL_HF_500 - a_HF_raw * x500
a_HF = a_HF_raw

# --- Plot
plt.figure()
plt.scatter(np.log10(df["Freq"]), df["NL"], s=10, label="SS3 data")

f = np.logspace(1, 5, 500)

def NL_turb(f): return a_turb*np.log10(f) + b_turb
def NL_LF(f):   return a_LF*np.log10(f) + b_LF
def NL_MF(f):   return a_MF*np.log10(f) + b_MF
def NL_HF(f):   return a_HF*np.log10(f) + b_HF

plt.plot(np.log10(f[f< f_t]), NL_turb(f[f < f_t]), label="Turbulence")
plt.plot(np.log10(f[(f>=f_t)&(f<200)]), NL_LF(f[(f>=f_t)&(f<200)]), label="LF")
plt.plot(np.log10(f[(f>=200)&(f<=500)]), NL_MF(f[(f>=200)&(f<=500)]), label="MF")
plt.plot(np.log10(f[f>=500]), NL_HF(f[f>=500]), label="HF")

plt.xlabel("log10(frequency [Hz])")
plt.ylabel("Noise Level [dB]")
plt.legend()
plt.title("SS3 Parametric Model vs Digitised Data")
plt.show()

This script reproduces all coefficients in Sections 7–10 and produces the verification figure described in Section 12.